==================================================

FXPR4.DOC  08/22/83   T. E. McCormick

==================================================

FXPR4 is a printer setup utility program for the

Epson FX-80 (160 cps) printer.	It is useful for

changing the character fonts, boldness, pitch,

vertical lines per inch, etc. quickly with a single

command line such as  FXPR4 RESET P12 LEFTM 5 SKIP.

Gary Novosielski wrote PRINTER.C (c) 1982 for the

MX-80.	This program was derived from that one,  and

Gary's approach was simple extended to other features,

and some minor changes were made to convert it from

CP/M BDS C to IBM PC DOS 2.0 Lattice C.  Both authors

have copyrighted their versions, and sale or commercial

use is forbidden without the authors' consent in writing.

The program is offered by the authors without charge

to individuals for their personal use only.

If you have an MX with graftrax plus, you may

be able to use this utility for most functions.

The purpose of this program is quickly set several

options with a single command from the system prompt.

PURPOSE OF THE PROGRAM:

From a single command, several printer

option commands may be sent to the FX-80.

The Epson printers have an extremely wide

range of software selectable print font sizes and

styles (called modes).	Line width, characters per

inch, lines per vertical inch, print intensity,

tab stops, style (pica, elite, italics, etc.),

left/right margin settings, and "global" commands

to perform several commonly used settings such as

for letter writing in pica, elite or proportional

can readily be set with a quick keyboard entry.

It is clearly not convenient to go into

BASIC or any other indirect means of sending those

commands out the printer port.	This program gets

the job done with a single command.

The Epxon FX-80 includes the "graftrak" option as

standard, and implements some new features in

addition to the 160 charater per second speed. Pin

addressing commands, and many advanced features are

not included in this program but you may be add them

since the source file is available.

FXPR4 will compile under  Lattice C ver 1.03 and

IBM PC DOS version 2.0.

Simply entering  FXPR4  will cause a menu

of options to be displayed.  These can be used as

parameters following  FXPR4  the next time it is

entered.

That menu appears as:

FXPR4 for Epson FX  (c) Tom McCormick  05/01/83

from PRINTER.C (c) 1982 G.P. Novosielski

FXPR4 <opt> [<opt>.....]   where <opt> may be:

RESET	   QUIET       PITCH 10    PITCH 17	DOC

LPI3	   LPI4        LPI5	   LPI6 	DOC1

LPI8	   LPI10       LPI12	   PLINCH 1-22	DOC1S

WIDTH 80 [....or any value between 2 and 137 ]	DOC8

SKIP	   NOSKIP      EJECT	   TABS n n n	DOC8S

PROPOR	   DRAFT       TEXT

PICA	   ELITE       ITALIC	   LEFTM  n

TINY	   TINYER      ROMAN	   RIGHTM n

LTRITA	   LTRITB      LTRELI

EMPHAS	   CONDEN      ENLARG	   DOUBLE

XEMPHA	   XCONDE      XENLAR	   XDOUBL

Various pitches (CPI) and bold or very bold...

P5	P6	      P10     P12     P17

P5B	P6B    P9B    P10B    P12B    P17B

P5VB	P6VB   P9VB   P10VB

\n  Example: FXPR4 RESET LPI6 PITCH 10 ELITE

There are more than 50 keyword options, and God knows how many combinations.

They have NOT all been tested !!  Be aware that the FX-80 treats various

combinations as toggles, or serially.  Some features have priorities

with regard to others, and some negate others.	The 150+ page manual has

several tables that will help, but you will have to experiment a little too.

It is a good idea to use RESET as your first keyword to "baseline" the

subsequent commands.  If you don't, you may simply be adding new parameters

to the previous mix of them.

The "RESET" command mentioned above will reset ALL modes to the power-on

configuration you have chosen via the DIP switches at installation.

FXPR4  PROGRAM OPTIONS:	ENTER ONE OR MORE

CONDEN	Condensed print mode

CONDEN ENLARG Condensed/enlarged mode at 6 characters per inch.

DOC	Global: RESET LPI6 ELITE LEFTM 8 RIGHTM 95 DOUBLE

DOC1	Global: RESET LPI6 ELITE LEFTM 8 RIGHTM 95 (single intensity)

DOC1S	Global: RESET LPI6 ELITE LEFTM 8 RIGHTM 95 skips over perf

DOC8	Global: RESET  LPI8 ELITE LEFTM 8 RIGHTM 95 eight lines per in

DOC8S	Global: RESET  LPI8 ELITE LEFTM 8 RIGHTM 95 skips over perf

DOUBLE	Double-strike mode: strikes each char twice to darken.

DRAFT	Cancels any double-strike mode, or super/sub scripting.

EJECT	Sends printer to top of next page.

ELITE	Elite type style

EMPHAS	Emphasized printing

ENLARG	Large lettering

ITALIC	Print in Italic style.

LEFTM	LEFTM n   sets left  margin to n column position

LPI	LPI n  (where n = 3 to 216) will cause the vertical

spacing to be n/216 inches per line......

7 LPI = 31	  9 LPI = 24   11 LPI = 20.

LPI3	Three  lines per inch vertical spacing

LPI4	Four   lines per inch vertical spacing

LPI5	Five   lines per inch vertical spacing

LPI6	Six    lines per inch vertical spacing

LPI8	Eight  lines per inch vertical spacing

LPI10	Ten    lines per inch vertical spacing

LPI12	Twelve lines per inch vertical spacing

LTRELI	Global command for RESET ELITE	P12  WIDTH 132	Normal

LTRITA	Global command for RESET ITALIC P12  WIDTH 132	Normal

LTRITB	Global command for RESET ITALIC P12B WIDTH 132	Bold

NOSKIP	The perforation will not be skipped over with blank lines.

P5	Pitch 5  characters to an inch horizontally

P5B	Pitch 5  and bold

P5VB	Pitch 5  and very bold

P6	Pitch 6  characters to an inch horizontally

P6B	Pitch 6  and bold

P6VB	Pitch 6  and very bold

P9B	Pitch 9  and bold  (Note: there is no P9)

P9VB	Pitch 9  and very bold

P10	Pitch 10 and normal intensity

P10B	Pitch 10 and bold

P10VB	Pitch 10 and very bold

P12	Pitch 12 and normal intensity

P12B	Pitch 12 and bold

P17	Pitch 17 and normal intensity

P17B	Pitch 17 and bold

PICA	Pica type style

PITCH	Characters per inch:  PITCH 10	or  PITCH 17 are valid.

PLINCH	Set page length in inches ( 1 - 22 ); PLINCH 11

PROPOR	Proportional spacing of letters (horizontally).

QUIET	Slow to 80 char per second (half speed) to quiet operation.

RESET	Cause printer to return to power-on option settings.

RIGHTM	RIGHTM n  sets right margin to n column position

ROMAN	Print in Roman	style.

SKIP	Skip over perforation specifying 6 lines.

TABS	Sets horizontal tab positions:	TAB n n n

TEXT	From original program.

TINY	Subscript sized, 10 lines per inch,  93 lines per 11" page,

skips over perf leaving one inch bottom margin.

LEFTM / RIGHTM may be used after TINY.

TINYER	Subscript sized, 12 lines per inch, 108 lines per 11" page,

skips over perf leaving one inch bottom margin.

LEFTM / RIGHTM may be used after TINYER.

WIDTH	WIDTH 80  or  [..any value between 2 and 137 ].

XCONDE	Cancels condensed mode.

XDOUBL	Cancels double-strike mode.

XEMPHA	Cancels emphasized mode.

XENLAR	Cancels enlarged mode.

The following print lines are useful in various combinations

to test FXPR4x after your modifications.....................

LINETEST

1	   2	     3	       4	 5	   6	     7

12345678901234567890123456789012345678901234567890123456789012345678901234567890

+++++++++1+++++++++2+++++++++3+++++++++4+++++++++5+++++++++6+++++++++7++++++++++++++++++++++++96+++++++++++++++++++++120+++++++++132  137

ABCDE abcde @#$%&?

ABC abc @#$

XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

This is lllllower case. 1111111111

THIS IS UUUUUPPER CASE. 2222222222

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII

iiiiiiiiiiiiiiiiiiiiiiiiiiiiiiiiii

wwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwww

WWWWWWWWWWWWWWWWWWWWWWWWWWWWWWWWWW

All lines have same # characters..

=-=-=-=-=-=-=-=-=-= end of FXPR4.DOC =-=-=-=-=-=-=-=-=-=-=

###############################################################################################################