SWAP PRINTERS COMMAND

Written by Thomas M. Rowlett

The Swap Printers (SWPTR) Command provides the capability to logically

swap the printer defined as LPT1: with the printer defined as LPT2:.  This

can be very useful to IBM PC owners who have printers connected to the two

(2) parallel ports of their PCs.  I developed the assembly language program

so that I could easily use either one of my two printers with many of my

application programs that use LPT1: exclusively, without having to recable

the printers or use an A-B Switch Box every time I wanted to swap them.

An example would be if one were using WordStar to print a draft of a

document using a dot matrix printer, then a final copy is desired using a

letter quality printer. Normally, one would have to disconnect the dot

matrix printer and connect the letter quality printer to the LPT1: port.  By

having a second printer port (LPT2:) and using the SWPTR command from

WordStar (Run a Program option from the main menu of WordStar), one can

logically swap LPT1: with LPT2: so that the output directed to LPT1: is

printed on the printer attached to LPT2:.  No cables to disconnect and

reconnect!  No pins bent or broken!  Just remain in the comfort of your

chair and let the computer do the printer swapping.  When it is desired to

swap the printers back to their original assignments, just run SWPTR again.

SWPTR initializes both printers when a swap takes place.  This was

necessary because some printers would not operate after the swap.  An

optional input of an "i" or "I" in the command line  (for example, "SWPTR

I") specifies that only an "inspect" of the printer assignment is desired.

Like all DOS Commands, upper or lower case letters may be used.  Executing

the inspect option does not cause either printer to be initialized.

SWPTR, when linked, becomes a COMmand file of DOS so it can be executed

like the DOS Commands MODE or DISKCOPY.  This usually requires executing

SWPTR before the application program.  Some programs, like WordStar, provide

a way to execute a DOS Command without exiting.  SWPTR works very well from

those programs.

Since the time I first developed this command, I have seen several

examples in other publications on how to make use of two printers connected

to the same IBM PC.  All the articles proposed using a BASIC program to do

the swapping.  I feel this assembly language program, SWPTR, offers much

more flexibility than the BASIC programs because it operates as a DOS

Command.

Please enjoy the flexibility offered by SWPTR.  You no longer have to

swap printer cables or connectors to be able to direct your printer output

to LPT2:  instead of LPT1:.

#nger have to

swap printer cables or connectors to be able to direct your printer output

to LPT2:  instea